## ART I – Exam Questions (30 MCQs)

### Question 1

According to the Von Neumann architecture, which statement is correct?

- A. Instructions and data are stored in two separate memories.

- B. Instructions are stored only in ROM.

- C. Instructions and data are stored in the same read-write memory.

- D. Data cannot be addressed in memory.

### Question 2

In the Von Neumann architecture, how does the CPU execute a program?

- A. It executes all instructions simultaneously.

- B. It executes instructions sequentially unless the execution sequence is modified.

- C. It executes instructions based on priority.

- D. It executes instructions randomly.

### Question 3

What is a Hardwired Program?

- A. A program stored on an SSD.

- B. A program implemented by physically connecting hardware components.

- C. A program written in Assembly language.

- D. BIOS firmware.

### Question 4

How is Software different from a Hardwired Program?

- A. Software is changed by loading a new sequence of instructions.

- B. Software requires rewiring hardware circuits.

- C. Software does not use the CPU.

- D. Software is stored only in RAM.

### Question 5

Which two major components make up the CPU?

- A. Memory and Cache.

- B. Instruction Interpreter and Arithmetic Logic Unit (ALU).

- C. ALU and Main Memory.

- D. Cache and Register File.

### Question 6

Which module converts data from external devices into signals the CPU can understand?

- A. Output Module.

- B. Instruction Decoder.

- C. Input Module.

- D. Cache Module.

### Question 7

What is the function of the Memory Address Register (MAR)?

- A. Stores the current instruction.

- B. Stores the memory address for the next read or write operation.

- C. Stores data read from memory.

- D. Stores the operation code (opcode).

### Question 8

What is the function of the Memory Buffer Register (MBR)?

- A. Stores the opcode.

- B. Stores the address of the next instruction.

- C. Stores data read from memory or data to be written into memory.

- D. Stores interrupt information.

### Question 9

What is the purpose of the I/O Address Register (I/OAR)?

- A. Identifies a specific I/O device.

- B. Identifies a memory address.

- C. Stores I/O data.

- D. Stores an instruction.

### Question 10

Which register contains the address of the next instruction to be fetched?

- A. Instruction Register (IR).

- B. Program Counter (PC).

- C. Accumulator (AC).

- D. Memory Address Register (MAR).

### Question 11

The Fetch Cycle ends when:

- A. The ALU completes an arithmetic operation.

- B. The fetched instruction is loaded into the Instruction Register (IR).

- C. The instruction is written into memory.

- D. The operand is fetched.

### Question 12

What does the Instruction Register (IR) store?

- A. Operand data.

- B. Memory address.

- C. The instruction that has just been fetched.

- D. Interrupt vector.

### Question 13

An Instruction Cycle consists of which two basic stages?

- A. Decode and Store.

- B. Fetch and Execute.

- C. Fetch and Interrupt.

- D. Read and Write.

### Question 14

Which of the following is NOT one of the four categories of instruction actions?

- A. Processor–Memory.

- B. Processor–I/O.

- C. Data Processing.

- D. Memory–Memory.

### Question 15

A Control instruction is used to:

- A. Write data into memory.

- B. Change the sequence of program execution.

- C. Perform arithmetic operations.

- D. Read data from an I/O device.

### Question 16

In the memory addition example, what is the purpose of the Accumulator (AC)?

- A. Stores memory addresses.

- B. Stores intermediate and final computation results.

- C. Stores opcodes.

- D. Stores interrupt vectors.

### Question 17

In the instruction cycle state diagram, IOD stands for:

- A. Instruction Operand Data.

- B. Instruction Operation Decoding.

- C. Interrupt Operation Detection.

- D. Internal Operand Decode.

### Question 18

The Operand Address Calculation (OAC) state is responsible for:

- A. Fetching an instruction.

- B. Decoding an instruction.

- C. Determining the address of an operand.

- D. Storing computation results.

### Question 19

In the instruction cycle state diagram, OS stands for:

- A. Operating System.

- B. Operand Store.

- C. Operand Select.

- D. Opcode Storage.

### Question 20

Interrupts improve processor performance mainly because:

- A. They increase CPU clock speed.

- B. They allow the CPU to perform other tasks while waiting for I/O devices.

- C. They make memory faster.

- D. They enable the ALU to operate in parallel.

### Question 21

Which type of interrupt is generated when a program attempts to divide by zero?

- A. Timer Interrupt.

- B. I/O Interrupt.

- C. Program Interrupt.

- D. Hardware Failure Interrupt.

### Question 22

A Timer Interrupt is commonly used to:

- A. Report RAM errors.

- B. Allow the operating system to perform periodic tasks.

- C. Transfer network data.

- D. Control cache memory.

### Question 23

When an interrupt occurs, what is the first action performed by the CPU?

- A. Clear the current instruction.

- B. Save the context of the currently executing program.

- C. Reset the Program Counter.

- D. Write results into memory.

### Question 24

An Interrupt Handler is typically part of the:

- A. BIOS.

- B. Operating System.

- C. Cache Controller.

- D. Compiler.

### Question 25

What is the main function of Direct Memory Access (DMA)?

- A. Increase ALU processing speed.

- B. Allow an I/O module to transfer data directly to and from memory.

- C. Allow memory to access the CPU directly.

- D. Skip instruction execution.

### Question 26

A System Bus consists of which three functional buses?

- A. Data Bus, Address Bus, and Control Bus.

- B. Memory Bus, Cache Bus, and PCI Bus.

- C. Read Bus, Write Bus, and Clock Bus.

- D. Input Bus, Output Bus, and DMA Bus.

### Question 27

The width of the Data Bus determines:

- A. The maximum RAM capacity.

- B. The number of bits transferred in one bus operation.

- C. The number of I/O devices supported.

- D. The CPU clock frequency.

### Question 28

The width of the Address Bus directly affects:

- A. I/O bandwidth.

- B. The maximum addressable memory capacity.

- C. Cache size.

- D. Register size.

### Question 29

What is the main advantage of a Point-to-Point Interconnect over a Shared Bus?

- A. It uses a shared communication medium.

- B. It eliminates the need for shared bus arbitration.

- C. Only one device can transmit at a time.

- D. It is used only in embedded systems.

### Question 30

In the QuickPath Interconnect (QPI) architecture, what is the unit of transfer at the Link Layer?

- A. Packet.

- B. Phit.

- C. Flit.

- D. Frame.

| Q | Ans | Q | Ans |
| --- | --- | --- | --- |
| 1 | C | 16 | B |
| 2 | B | 17 | B |
| 3 | B | 18 | C |
| 4 | A | 19 | B |
| 5 | B | 20 | B |
| 6 | C | 21 | C |
| 7 | B | 22 | B |
| 8 | C | 23 | B |
| 9 | A | 24 | B |
| 10 | B | 25 | B |
| 11 | B | 26 | A |
| 12 | C | 27 | B |
| 13 | B | 28 | B |
| 14 | D | 29 | B |
| 15 | B | 30 | C |